Architectures for Embedded Systems

- An embedded system is a computer system with a dedicated function within a larger mechanical or electrical system, often with real-time computing constraints.

- An embedded system has specific requirements and performs pre-defined tasks, unlike a general-purpose personal computer. An embedded system is a programmed hardware device. A programmable hardware chip is the platform and it is programmed with particular applications. Embedded systems are a combination of hardware and software which facilitates mass production and a host of applications.

- A processor is an important unit in the embedded system hardware. It is the heart of the embedded system.

- Embedded systems contain processing cores that are either microcontrollers or DSP. Embedded Systems are the crucial components of the modern compacted devices with multifunction capabilities.

- Embedded systems are specific computer programs that combine the functions of specially designed software’s and hardware’s and are completely encapsulated by the devices that they control.

- The key characteristic, however, is being dedicated to handle a particular task. Since the embedded system is dedicated to specific tasks, design engineers can optimize it to reduce the size and cost of the product and increase the reliability and performance. Some embedded systems are mass-produced, benefiting from economies of scale.

- Physically, embedded systems range from portable devices such as digital watches and MP3 players, PDAs, to large stationary installations like traffic lights, factory controllers, and largely complex systems like hybrid vehicles, MRI, and avionics. Complexity varies from low, with a single microcontroller chip, to very high with multiple units, peripherals and networks mounted inside a large chassis or enclosure.

- The program instructions written for embedded systems are referred to as firmware, and are stored in read-only memory or Flash memory chips. They run with limited computer hardware resources: little memory, small or non-existent keyboard or screen.

- Compared to desktop and server systems, embedded systems have a much wider range of processing power and cost—from systems containing low-end 8-bit and 16-bit processors that may cost less than a dollar, to those  containing full 32-bit microprocessors capable of operating in the 500 MIPS range that cost approximately 10 dollars, to those containing high-end embedded processors that cost hundreds of dollars and can execute several billions of instructions per second.

- Although the range of computing power in the embedded systems market is very large, price is a key factor in the design of computers for this space. Performance requirements do exist, of course, but the primary goal is often meeting the performance need at a minimum price, rather than achieving higher performance at a higher price.

- Embedded systems often process information in very different ways from general-purpose processors. Typically these applications include deadline-driven constraints—so-called real-time constraints. In these applications, a particular computation must be completed by a certain time or the system fails.

- Embedded systems applications typically involve processing information as signals. The lay term “signal” often connotes radio transmission, and that is true for some embedded systems (e.g., cell phones). But a signal may be an image, a motion picture composed of a series of images, a control sensor measurement, and so on. Signal processing requires specific computation that many embedded processors are optimized for. We discuss this in depth below.

- In many embedded applications, the memory can be a substantial portion of the system cost, and it is important to optimize memory size in such cases. Sometimes the application is expected to fit entirely in the memory on the processor chip; other times the application needs to fit in its entirety in a small, off-chip memory. In either case, the importance of memory size translates to an emphasis on code size, since data size is dictated by the application. Some architectures have special instruction set capabilities to reduce code size.

- Larger memories also mean more power, and optimizing power is often critical in embedded applications. Although the emphasis on low power is frequently driven by the use of batteries, the need to use less expensive packaging and the absence of a fan for cooling also limit total power consumption.

- In practice, embedded problems are usually solved by one of three approaches:

1. The designer uses a combined hardware/software solution that includes some custom hardware and an embedded processor core that is integrated with the custom hardware, often on the same chip.

2. The designer uses custom software running on an off-the-shelf embedded  processor.

3. The designer uses a digital signal processor and custom software for the processor. Digital signal processors are processors specially tailored for signal processing applications. We discuss some of the important differences between digital signal processors and general-purpose embedded processors below.

Feature Embedded Desktop Server

Price of system $1000–$10,000 $10,000–$10,000,000 $10–$100,000 (including network routers at the high end)

Price of microprocessor $100–$1000 $200–$2000 $0.20–$200 (per processor) Module (per processor)

Microprocessors sold per 150,000,000 4,000,000 300,000,000 (32-bit and 64- year (estimates for 2000) bit processors only)

Critical system design Issues Price-performance Throughput, availability, Price, power consumption, graphics performance Scalability application-specific performance

Figure E.1 A summary of the three computing classes and their system characteristics.

Real-Time Processing

- The performance requirement in an embedded application is a real-time requirement. A real-time performance requirement is one where a segment of the application has an absolute maximum execution time that is allowed.

- Real-time performance tends to be highly application dependent. It is usually measured using kernels either from the application or from a standardized benchmark The construction of a hard real-time system involves three key variables.

  - The first is the rate at which a particular task must occur.

  - Coupled to this are the hardware and software required to achieve that real-time rate. Often, structures that are very advantageous on the desktop are the enemy of hard real-time analysis. For example, branch speculation, cache memories, and so on introduce uncertainty into code. A particular sequence of code may execute either very efficiently or very inefficiently, depending on whether the hardware branch predictors and caches “do their jobs.” Engineers must analyze code assuming the worst-case execution time (WCET). In the case of traditional microprocessor hardware, if one assumes that all branches are mis predicted and all caches miss, the WCET is overly pessimistic. Thus, the system designer may end up overdesigning a system to achieve a given WCET, when a much less expensive system would have sufficed.

  - In order to address the challenges of hard real-time systems, and yet still exploit such well-known architectural properties as branch behavior and access locality, it is possible to change how a processor is designed. Consider branch prediction: Although dynamic branch prediction is known to perform far more accurately than static “hint bits” added to branch instructions, the behavior of static hints is much more predictable. Furthermore, although caches perform better than software-managed on-chip memories, the latter produces predictable memory latencies. In some embedded processors, caches can be converted into software-managed on-chip memories via line locking. In this approach, a cache line can be locked in the cache so that it cannot be replaced until the line is unlocked.

# Digital signal processor

- A digital signal processor (DSP) is a specialized microprocessor with an architecture optimized for the operational needs of digital signal processing.

# Fig: A typical digital processing system

# Characteristics:

- Digital signal processing algorithms typically require a large number of mathematical operations to be performed quickly and repeatedly on a series of data samples.

- Signals (perhaps from audio or video sensors) are constantly converted from analog to digital, manipulated digitally, and then converted back to analog form.

- Many DSP applications have constraints on latency; that is, for the system to work, the DSP operation must be completed within some fixed time, and deferred (or batch) processing is not viable.

- Most general-purpose microprocessors and operating systems can execute DSP algorithms successfully, but are not suitable for use in portable devices such as mobile phones and PDAs because of power supply and space constraints

- A specialized digital signal processor, however, will tend to provide a lower-cost solution, with better performance, lower latency, and no requirements for specialized cooling or large batteries.

# Architecture:

- By the standards of general-purpose processors, DSP instruction sets are often highly irregular. One implication for software architecture is that hand-optimized assembly-code routines are commonly packaged into libraries for re-use, instead of relying on advanced compiler technologies to handle essential algorithms.

Hardware features visible through DSP instruction sets commonly include:

- Hardware modulo addressing, allowing circular buffers to be implemented without having to constantly test for wrapping.

- A memory architecture designed for streaming data, using DMA extensively and expecting code to be written to know about cache hierarchies and the associated delays.

- Driving multiple arithmetic units may require memory architectures to support several accesses per instruction cycle

- Separate program and data memories (Harvard architecture), and sometimes concurrent access on multiple data busses

- Special SIMD (single instruction, multiple data) operations

- Some processors use VLIW techniques so each instruction drives multiple arithmetic units in parallel

- Special arithmetic operations, such as fast multiply–accumulates (MACs). Many fundamental DSP algorithms, such as FIR filters or the Fast Fourier transform (FFT) depend heavily on multiply–accumulate performance.

- Bit-reversed addressing, a special addressing mode useful for calculating FFTs

- Special loop controls, such as architectural support for executing a few instruction words in a very tight loop without overhead for instruction fetches or exit testing

- Deliberate exclusion of a memory management unit. DSPs frequently use multi-tasking operating systems, but have no support for virtual memory or memory protection. Operating systems that use virtual memory require more time for context switching among processes, which increases latency.

### Program flow

- Floating-point unit integrated directly into the data path

- Pipelined architecture

- Highly parallel multiplier–accumulators (MAC units)

- Hardware-controlled looping, to reduce or eliminate the overhead required for looping operations

### Memory architecture

- DSPs often use special memory architectures that are able to fetch multiple data and/or instructions at the same time:

  - Harvard architecture

  - Modified von Neumann architecture

- Use of direct memory access

- Memory-address calculation unit

### Data operations

- Saturation arithmetic, in which operations that produce overflows will accumulate at the maximum (or minimum) values that the register can hold rather than wrapping around (maximum+1 doesn't overflow to minimum as in many general-purpose CPUs, instead it stays at maximum). Sometimes various sticky bits operation modes are available.

- Fixed-point arithmetic is often used to speed up arithmetic processing

- Single-cycle operations to increase the benefits of pipelining

### Instruction sets

- Multiply–accumulate (MAC, including fused multiply–add, FMA) operations, which are used extensively in all kinds of matrix operations, such as convolution for filtering, dot product, or even polynomial evaluation (see Horner scheme)

- Instructions to increase parallelism: SIMD, VLIW, superscalar architecture

- Specialized instructions for modulo addressing in ring buffers and bit-reversed addressing mode for FFT cross-referencing

- Digital signal processors sometimes use time-stationary encoding to simplify hardware and increase coding efficiency.

Signal Processing and Embedded Applications: The Digital Signal Processor

- A digital signal processor (DSP) is a special-purpose processor optimized for executing digital signal processing algorithms. Most of these algorithms, from time-domain filtering (e.g., infinite impulse response and finite impulse response filtering), to convolution, to transforms (e.g., fast Fourier transform, discrete cosine transform), to even forward error correction (FEC) encodings, all have as their kernel the same operation: a multiply-accumulate operation.

- To accelerate this, DSPs typically feature special-purpose hardware to perform Multiply-Accumulate (MAC). A MAC instruction of               “MAC A,B,C”  has the semantics of “A = A + B * C.” In some situations, the  performance of this operation is so critical that a DSP is selected for an application based solely upon its MAC operation throughput. DSPs often employ fixed-point arithmetic. If you think of integers as having a binary point to the right of the least-significant bit, fixed point has a binary point just to the right of the sign bit. Hence, fixed-point data are fractions between –1 and +1.

- Fixed point can be thought of as a low-cost floating point. It doesn’t include an exponent in every word and doesn’t have hardware that automatically aligns and normalizes operands. Instead, fixed point relies on the DSP programmer to keep the exponent in a separate variable and ensure that each result is shifted left or right to keep the answer aligned to that variable.

- Since this exponent variable is often shared by a set of fixed-point variables, this style of arithmetic is also called blocked floating point, since a block of variables has a common exponent. To support such manual calculations, DSPs usually have some registers that are wider to guard against round-off error, just as  floating-point units internally have extra guard bits. Figure E.2 surveys four generations of DSPs, listing data sizes and width of the accumulating registers. Note that DSP architects are not bound by the powers of 2 for word sizes. Figure E.3 shows the size of data operands for the TI TMS320C55 DSP.

- In addition to MAC operations, DSPs often also have operations to accelerate portions of communications algorithms. An important class of these algorithms revolve around encoding and decoding forward error correction codes—codes in which extra information is added to the digital bit stream to guard against errors in transmission. A code of rate m/n has m information bits for (m + n) check bits. So, for example, a 1/2 rate code would have 1 information bit per every 2 bits.

Figure E.2 Four generations of DSPs, their data width, and the width of the registers that reduces round-off error.

Figure E.3 Size of data operands for the TMS320C55 DSP.

- About 90% of operands are 16 bits. This DSP has two 40-bit accumulators. There are no floating-point operations, as is typical of many DSPs, so these data are all fixed-point integers.

- Such codes are often called trellis codes because one popular graphical flow diagram of their encoding resembles a garden trellis. A common algorithm for decoding trellis codes is due to Viterbi. This algorithm requires a sequence of compares and selects in order to recover a transmitted bit’s true value. Thus DSPs often have compare-select operations to support Viterbi decode for FEC codes.

The TI 320C55

- At one end of the DSP spectrum is the TI 320C55 architecture. The C55 is optimized for low-power, embedded applications. Its overall architecture is shown in Figure E.4.

- At the heart of it, the C55 is a seven-staged pipelined CPU. The stages are outlined below:

- Fetch stage reads program data from memory into the instruction buffer queue.

- Decode stage decodes instructions and dispatches tasks to the other primary functional units.

- Address stage computes addresses for data accesses and branch addresses for program discontinuities.

- Access 1/Access 2 stages send data read addresses to memory.

- Read stage transfers operand data on the B bus, C bus, and D bus.

- Execute stage executes operation in the A unit and D unit and

- performs writes on the E bus and F bus.

Figure E.4 Architecture of the TMS320C55 DSP

- The C55 pipeline performs pipeline hazard detection and will stall on write after read (WAR) and read after write (RAW) hazards.

- The C55 does have a 24 KB instruction cache, but it is configurable to support various workloads. It may be configured to be two-way set associative, direct-mapped, or as a “ramset.” This latter mode is a way to support hard real time applications. In this mode, blocks in the cache cannot be replaced.

- The C55 also has advanced power management. It allows dynamic power management through software-programmable “idle domains.” Blocks of circuitry on the device are organized into these idle domains. Each domain can operate normally or can be placed in a low-power idle state. A programmer accessible Idle Control Register (ICR) determines which domains will be placed in the idle state when the execution of the next IDLE instruction occurs.

- The six domains are CPU, direct memory access (DMA), peripherals, clock generator,  instruction cache, and external memory interface. When each domain is in the idle state, the functions of that particular domain are not available. However, in the peripheral domain, each peripheral has an Idle Enable bit that controls whether or not the peripheral will respond to the changes in the idle state.

- Thus, peripherals can be individually configured to idle or remain active when the peripheral domain is idled. Since the C55 is a DSP, the central feature is its MAC units.

- The C55 has two MAC units, each comprised of a 17-bit by 17-bit multiplier coupled to a 40-bit dedicated adder. Each MAC unit performs its work in a single cycle; thus, the C55 can execute two MACs per cycle in full pipelined operation. This kind of capability is critical for efficiently performing signal processing applications. The C55 also has a compare, select, and store unit (CSSU) for the add/compare section of the Viterbi decoder.

The TI 320C6x

- In stark contrast to the C55 DSP family is the high-end Texas Instruments VelociTI 320C6x family of processors. The C6x processors are closer to traditional very long instruction word (VLIW) processors because they seek to exploit the high levels of instruction-level parallelism (ILP) in many signal processing algorithms.

- Texas Instruments is not alone in selecting VLIW for exploiting ILP in the embedded space. Other VLIW DSP vendors include Ceva, StarCore, Philips/ Tri Media, and STMicroelectronics.

- Why do these vendors favor VLIW over superscalar? For the embedded space, code compatibility is less of a problem, and so new applications can be either hand tuned or recompiled for the newest generation of processor. The other reason superscalar excels on the desktop is because the compiler cannot predict memory latencies at compile time.

- In embedded, however, memory latencies are often much more predictable. In fact, hard real-time constraints force memory latencies to be statically predictable. Ofcourse, a superscalar would also perform well in this environment with these constraints, but the extra hardware to dynamically schedule instructions is both wasteful in terms of precious chip area and in terms of power consumption. Thus VLIW is a natural choice for high-performance embedded.

- The C6x family employs different pipeline depths depending on the family member. For the C64x, for example, the pipeline has 11 stages. The first four stages of the pipeline perform instruction fetch, followed by two stages for instruction decode, and finally four stages for instruction execution. The overall architecture of the C64x is shown below in Figure E.6.

- The C6x family’s execution stage is divided into two parts, the left or “1” side and the right or “2” side.

- The L1 and L2 units perform logical and arithmetic operations. D units in contrast perform a subset of logical and arithmetic operations but also perform memory acesses (loads and stores).

- The two M units perform multiplication and related operations (e.g., shifts). Finally the S units perform comparisons, branches, and some SIMD.

- Each side has its own 32- entry, 32-bit register file (the A file for the 1 side, the B file for the 2 side). A side may access the other side’s registers, but with a 1- cycle penalty. Thus, an instruction executing on side 1 may access B5, for example, but it will take 1- cycle extra to execute because of this.

- VLIWs are traditionally very bad when it comes to code size, which runs contrary to the needs of embedded systems. However, the C6x family’s approach “compresses” instructions, allowing the VLIW code to achieve the same density as equivalent RISC code. To do so, instruction fetch is carried out on an “instruction packet,” shown in Figure E.5. Each instruction has a p bit that specifies whether this instruction is a member of the current VLIW word or the next VLIW word (see the figure for a detailed explanation). Thus, there are now no NOPs that are needed for VLIW encoding.

- Software pipelining is an important technique for achieving high performance in a VLIW. But software pipelining relies on each iteration of the loop having an identical schedule to all other iterations. Because conditional branch instructions disrupt this pattern, the C6x family provides a means to conditionally execute instructions using predication.

- In predication, the instruction performs its work. But when it is done executing, an additional register, for example A1, is checked. If A1 is zero, the instruction does not write its results. If A1 is nonzero, the instruction proceeds normally. This allows simple if-then and if-then-else structures to be collapsed into straight-line code for software pipelining.

- Figure E.5 Instruction packet of the TMS320C6x family of DSPs.

Figure E.6 Architecture of the TMS320C64x family of DSPs

The C6x is an eight-issue traditional VLIW processor

- The p bits determine whether an instruction begins a new VLIW word or not. If the p bit of instruction i is 1, then instruction i + 1 is to be executed in parallel with (in the same cycle as) instruction i. If the p bit of instruction i is 0, then instruction i + 1 is executed in the cycle after instruction i.

Embedded Multiprocessors

Multiprocessors are now common in server environments, and several desktop multiprocessors are available from vendors, such as Sun, Compaq, and Apple. In the embedded space, a number of special-purpose designs have used customized multiprocessors, including the Sony PlayStation 2.

Many special-purpose embedded designs consist of a general-purpose programmable processor or DSP with special-purpose, finite-state machines that are used for stream-oriented I/O. In applications ranging from computer graphics and media processing to telecommunications, this style of special-purpose multiprocessor is becoming common. Although the interprocessor interactions in such  designs are highly regimented and relatively simple—consisting primarily of a simple communication channel—because much of the design is committed to silicon, ensuring that the communication protocols among the input/output processors and the general-purpose processor are correct is a major challenge in such designs.

More recently, we have seen the first appearance, in the embedded space, of embedded multiprocessors built from several general-purpose processors. These multiprocessors have been focused primarily on the high-end telecommunications and networking market, where scalability is critical. An example of such a design is the MXP processor designed by empowerTel Networks for use in voiceover- IP systems. The MXP processor consists of four main components:

- An interface to serial voice streams, including support for handling jitter.

- Support for fast packet routing and channel lookup

- A complete Ethernet interface, including the MAC layer

- Four MIPS32 R4000-class processors, each with its own cache (a total of 48 KB or 12 KB per processor)

The MIPS processors are used to run the code responsible for maintaining the voice-over-IP channels, including the assurance of quality of service, echo cancellation, simple compression, and packet encoding. Since the goal is to run

as many independent voice streams as possible, a multiprocessor is an ideal solution.

Because of the small size of the MIPS cores, the entire chip takes only 13.5M  transistors. Future generations of the chip are expected to handle more voice channels, as well as do more sophisticated echo cancellation, voice activity detection, and more sophisticated compression.

Multiprocessing is becoming widespread in the embedded computing arena for two primary reasons. First, the issues of binary software compatibility, which plague desktop and server systems, are less relevant in the embedded space. Often software in an embedded application is written from scratch for an application or significantly modified (note that this is also the reason VLIW is favored over superscalar in embedded instruction-level parallelism). Second, the applications often have natural parallelism, especially at the high end of the embedded space. Examples of this natural parallelism abound in applications such as a settop box, a network switch, a cell phone or a game system. The lower barriers to use of thread-level parallelism together with the greater sensitivity to die cost (and hence efficient use of silicon) are leading to widespread adoption of multiprocessing in the embedded space, as the application needs grow to demand more performance.